Android Auto

Android Auto 개요  |  Android for Cars  |  Android Developers

휴대전화를 연결하여 자동차 디스플레이에서 직접 Android 앱을 사용할 수 있음.

3.0 이후에는 플레이스토어에 등록된 앱만 연동 가능

3203217.3203278

USB 케이블을 이용한 유선 연결, Wifi, Bluetooth 무선 연결 옵션이 있음

비디오 재생은 금지되어 있음 in-vehicle infotainment (IVI) system

non-iOS devices exploit CarPlay compatibility

CarPlay employs the Network Control Model (NCM), Inter-Accessory Protocol 2 (iAP2), and dedicated CarPlay sessions to facilitate data exchange and application execution with the IVI.

Basic IVI adapters: Wifi/Bluetooth => advanced one: multimedia

the work [48] conducts a com prehensive security assessment of the MirrorLink protocol, revealing heap overflow vulnerabilities within the system.

An other work [59] investigates the remote control services of IVI systems and identifies potential attack scenarios, such as the extraction of authentication tokens.

Moreover, Automo tive Grade Linux has attracted significant research interest, leading to the discovery of 33 reproducible exploits [41].

In addition, hidden commands within Tesla’s infotainment system were identified through static analysis techniques [61].

Furthermore, DONGLESCOPE ana lyzes OBD-II dongles and describes five vulnerabilities [60].

Then, CANHUNTER extracts CAN bus commands from the companion applications of OBD-II dongles [62].

First, we exam ine how third-party CarPlay adapters establish connectivity with IVI systems even without non-IOS devices. Second, we systematically analyze the potential security risks posed by such devices

Apple Carplay

자동차 디스플레이에서 iPhone을 사용할 수 있는 방법

iOS 13과 14 이후 카테고리와 권한 증대

Apple CarPlay Hacking Risks: CVE-2025-24132 Explained | Oligo Security

Airborne Vurnerability

CVE-2025-24252 and CVE-2025-24132

expose weaknesses in AirPlay’s data handling, enabling remote code execution (RCE).

지도, 전화, 메시지, 음악, 주차 등의 옵션